Network Automation

Technical Documentation

Cisco Switch Configuration Automation via Serial/Console

Version 1.0  ·  April 2026

github.com/Paul447/NetworkAutomation

1. Overview

Network Automation is a Python-based toolkit that automates the configuration of Cisco IOS switches over a serial/console connection. Instead of manually entering commands into each device through a console cable, these scripts connect to the switch and push configuration automatically — setting hostnames, creating VLANs, configuring Switched Virtual Interfaces (SVIs), enabling port security, and saving the configuration.

The toolkit uses the netmiko library to handle all serial communication and IOS interaction, providing a clean Python interface over a raw console connection.

Key Capabilities

Layer 2 switch base configuration (hostname, VLANs, credentials, passwords)

Layer 3 switch configuration with full inter-VLAN routing via SVIs

Port security enforcement (MAC limiting, sticky learning, violation modes)

Serial port discovery utility

2. Requirements

Hardware

USB-to-serial console cable (e.g. USB-A to RJ45)

Cisco IOS switch with a physical console port

macOS computer (Linux also supported — see Section 4)

Software

| Component | Version | Purpose |
| --- | --- | --- |
| Python | 3.8+ | Runtime environment |
| netmiko | 4.6.0 | Cisco device automation over serial/SSH |
| pyserial | 3.5 | Serial port communication |
| paramiko | 4.0.0 | SSH protocol (netmiko dependency) |
| textfsm | 2.1.0 | IOS output parsing |

3. Installation

Step 1 — Clone the repository

git clone https://github.com/Paul447/NetworkAutomation.git

cd NetworkAutomation

Step 2 — Create a virtual environment

python3 -m venv venv

source venv/bin/activate       # macOS / Linux

# venv\Scripts\activate       # Windows

Step 3 — Install dependencies

pip install -r requirements.txt

4. Project Structure

NetworkAutomation/

│

├── configs/

│   ├── base_config.txt              # IOS commands applied to every L2 switch

│   └── README.md                    # Config template reference

│

├── scripts/

│   ├── find_serial_ports.py         # Discover available serial ports

│   ├── configure_l2_switch.py       # Layer 2 switch setup

│   ├── configure_l3_switch.py       # Layer 3 switch setup (inter-VLAN routing)

│   └── configure_port_security.py   # Switchport port security

│

├── requirements.txt

├── README.md

└── CONTRIBUTING.md

5. Connection Setup

5.1  Find Your Serial Port

Connect the USB-to-serial console cable to the switch, then run:

python scripts/find_serial_ports.py

Example output:

Available serial ports:

/dev/tty.usbserial-1130

/dev/tty.Bluetooth-Incoming-Port

Look for the entry containing usbserial. The number at the end (e.g. 1130) is unique to your adapter. Linux users will see /dev/ttyUSB0 or /dev/ttyS0 instead.

5.2  Update the Port in Your Script

Open your target script and update the port value in the device dictionary:

device = {

"device_type": "cisco_ios_serial",

"serial_settings": {

"port": "/dev/tty.usbserial-1130",   # <-- replace with your port

"baudrate": 9600,

},

}

5.3  Enable Credentials (if required)

If the switch has a username, password, or enable secret configured, uncomment and fill in the relevant lines:

device = {

...

"username": "admin",

"password": "YourPassword",

"secret":   "YourEnableSecret",

}

6. Scripts

6.1  find_serial_ports.py

Utility script that lists all serial/TTY ports currently detected on the system. Always run this first before any configuration script to identify the correct port name.

Usage

python scripts/find_serial_ports.py

6.2  configure_l2_switch.py

Automates base configuration for a Cisco Layer 2 switch. Connects to the device over console and performs the following steps in sequence:

Enters privileged exec mode (enable)

Prompts for a hostname and sets it on the device

Applies configs/base_config.txt — domain name, local credentials, console password, enable secret, and service password-encryption

Interactively creates VLANs (user provides VLAN ID, name, and description for each)

Saves the running configuration to NVRAM (write memory)

Usage

python scripts/configure_l2_switch.py

Example Session

Hostname for this device: CoreSwitch01

How many VLANs do you want to create? 2

Create VLAN 1? (y/n): y

VLAN ID: 10

VLAN name: MANAGEMENT

Description: Management VLAN

VLAN 10 (MANAGEMENT) created.

Create VLAN 2? (y/n): y

VLAN ID: 20

VLAN name: USERS

Description: User access VLAN

VLAN 20 (USERS) created.

Configuration saved successfully.

6.3  configure_l3_switch.py

Automates full configuration for a Cisco Layer 3 switch with inter-VLAN routing support. Layer 3 switches can route traffic between VLANs using Switched Virtual Interfaces (SVIs) — virtual Layer 3 interfaces assigned to each VLAN — eliminating the need for a separate router.

Steps performed:

Sets hostname and IP domain name

Enables ip routing globally (activates Layer 3 packet forwarding)

Creates VLANs with optional SVI configuration (IP address + subnet mask per VLAN interface)

Optionally configures a static default route to an upstream gateway

Saves the configuration

Usage

python scripts/configure_l3_switch.py

Example Session

Hostname for this device: DistroSwitch01

IP domain name (e.g. example.com): corp.local

IP routing enabled.

How many VLANs do you want to create? 1

Create VLAN 1? (y/n): y

VLAN ID: 10

VLAN name: MANAGEMENT

Configure SVI (IP address) for VLAN 10? (y/n): y

IP address (e.g. 192.168.10.1): 192.168.10.1

Subnet mask (e.g. 255.255.255.0): 255.255.255.0

Description: Management SVI

SVI for VLAN 10 configured: 192.168.10.1 255.255.255.0

Configure a static default route? (y/n): y

Next-hop IP address: 192.168.1.1

Default route via 192.168.1.1 configured.

Configuration saved successfully.

What is an SVI?

A Switched Virtual Interface is a virtual Layer 3 interface assigned to a VLAN. Assigning an IP address to it allows the switch to act as the default gateway for that VLAN and route traffic between VLANs without a separate router.

6.4  configure_port_security.py

Configures port security on one or more switchport interfaces to restrict which devices can connect to the network based on MAC address. Supports configuring multiple interfaces in a single run.

Settings Configured Per Interface

| IOS Command | Description |
| --- | --- |
| switchport mode access | Sets port to access mode (required for port security) |
| switchport port-security | Enables port security on the interface |
| port-security maximum <n> | Maximum MAC addresses allowed (default: 1) |
| port-security mac-address sticky | Switch auto-learns and persists connected MACs |
| port-security violation <mode> | Action taken when an unauthorized MAC is detected |
| port-security mac-address <MAC> | Optional: pin a specific trusted MAC address |

Violation Modes

| Mode | What Happens | Port Stays Up? | Log Entry? |
| --- | --- | --- | --- |
| shutdown | Port is err-disabled immediately | No | Yes |
| restrict | Unauthorized frames dropped | Yes | Yes |
| protect | Unauthorized frames dropped silently | Yes | No |

Usage

python scripts/configure_port_security.py

Example Session

Enter interface(s) to secure, comma-separated: fa0/1, fa0/2

--- Configuring port security on fa0/1 ---

Max allowed MAC addresses (default 1): 1

Violation modes: shutdown | restrict | protect

Violation mode (default: shutdown): shutdown

Pin a specific trusted MAC address? (leave blank to skip):

Port security applied to fa0/1: max=1, violation=shutdown, sticky=enabled.

--- Configuring port security on fa0/2 ---

Max allowed MAC addresses (default 1): 2

Violation mode (default: shutdown): restrict

Pin a specific trusted MAC address? (leave blank to skip): AA:BB:CC:DD:EE:FF

Pinning MAC AA:BB:CC:DD:EE:FF to fa0/2.

Port security applied to fa0/2: max=2, violation=restrict, sticky=enabled.

Port security configuration saved successfully.

7. Configuration Template  —  configs/base_config.txt

This file contains IOS commands applied line-by-line to the device by configure_l2_switch.py. Edit it before running the script to match your environment.

Default Contents

ip domain-name jackmid.lib.edu

username admin secret ABcd1234!

line console 0

password ABcd1234!

login

exit

enable secret ABcd1234!

service password-encryption

Command Reference

| Command | Purpose |
| --- | --- |
| ip domain-name <domain> | Sets the DNS domain. Required for SSH RSA key generation. |
| username admin secret <pass> | Creates a local admin account with an encrypted password. |
| line console 0 | Enters console line configuration mode. |
| password <pass> | Sets the console line password. |
| login | Enables password authentication on the console line. |
| enable secret <pass> | Sets the privileged exec password (MD5 encrypted). |
| service password-encryption | Encrypts all plaintext passwords in the running config. |

8. Troubleshooting

No serial ports found

Verify the USB-to-serial cable is physically connected to both the PC and the switch console port

Unplug and re-plug the cable, then re-run find_serial_ports.py

macOS may require a driver for your adapter chipset (CH340 or CP2102 are common — search the chip name + 'macOS driver')

netmiko hangs or times out on connect

Confirm the port name is correct by running find_serial_ports.py

Baud rate must be 9600 (Cisco default)

Cable must be in the CONSOLE port — not AUX or USB management port

Press Enter on the device before running the script to wake the console session

'Invalid input detected' error on device

The switch may be in ROMMON or setup wizard mode — boot it to the IOS prompt first

Confirm enable mode is accessible (correct secret in the device dict)

Check IOS version supports the commands being sent

Permission denied on serial port (Linux)

sudo usermod -aG dialout $USER

# Log out and back in for the change to take effect

Port security port stuck err-disabled

A violation occurred and the port was shut down (shutdown violation mode)

To re-enable: connect to the switch, go to the interface, type 'shutdown' then 'no shutdown'

Consider switching to 'restrict' mode if you don't want ports to go down on violations

9. Roadmap

Planned features and scripts for future development:

| Feature | Description |
| --- | --- |
| SSH hardening | Generate RSA keys, restrict VTY lines to SSH-only (transport input ssh) |
| Config backup & restore | Save show running-config to a timestamped file; push it back on demand |
| DHCP pool setup | Wizard for creating DHCP scopes on L3 switches |
| Trunk & EtherChannel | Automate 802.1Q trunk links and LACP port channels between switches |
| OSPF / EIGRP | Basic dynamic routing setup wizard |
| Linux / Windows support | Auto-detect OS and use /dev/ttyUSB* or COM* serial patterns |
| Dry-run / validation | Print commands to console without sending them to the device |

10. Contributing

Getting Started

# Fork the repo on GitHub, then clone your fork

git clone https://github.com/<your-username>/NetworkAutomation.git

cd NetworkAutomation

python3 -m venv venv && source venv/bin/activate

pip install -r requirements.txt

Script Standards

Module docstring at the top — purpose, numbered steps, requirements, usage

Section comments (# -- Section name ---) separating logical blocks

Inline comments for non-obvious IOS behaviour

conn.save_config() always called at the end

port marked # <-- update this in the device dictionary

Naming Conventions

| Item | Convention | Example |
| --- | --- | --- |
| Script files | snake_case, verb-prefixed | configure_l2_switch.py |
| Config files | snake_case | base_config.txt |
| Variables | snake_case | vlan_id, next_hop |
| Functions | snake_case, descriptive | configure_port_security_on_interface() |

PR Checklist

Script has a full module docstring

Section comments present for each logical block

conn.save_config() is called at the end

README.md updated if a new script was added

Tested on a real device or Packet Tracer / GNS3

────────────────────────────────────────────────────────────

Network Automation  ·  github.com/Paul447/NetworkAutomation  ·  April 2026